# LINT

RTL simulation validates functional intent under stimulus. Synthesis infers hardware structure from coding semantics.

Mismatch arises when:

- RTL behavior ≠ inferred gate-level hardware

- Power-up state undefined

- Async logic used unsafely

- FSM encoding incomplete

- Width truncation silent

- Latch inferred unintentionally

Lint performs static semantic analysis to predict how synthesis interprets RTL before netlist generation.

Lint ensures structural safety before clock-domain and property analysis.

VIOLATIONS BASED ON SEVERITY

FATAL VIOLATIONS (highest- severity violation): Structurally illegal or non-synthesizable
1. Syntax & Parsing Errors:

- Missing endmodule

- Unsupported SystemVerilog syntax

- Unmatched parentheses, begin-end blocks

- Incorrect port declarations

- Missing (,) and (;)

Fix: Set the LINT on classify based on severity, then see the FATAL it is showing , it will describe the problem with the file path and the LINT no where the issue exist. Go to that exact line (gvim command => ‘:line_no’) and resolve it.

2. Multiple Drivers on a Net/Reg/Reset/Clk:

Two or more sources driving the same net without resolution.

Risk: Contention, unintended wired logic.

Example:   assign a = x;

assign a = y;  // FATAL — multiple drivers

Fix: Make sure the Net should be driven by Single driver ONLY. Or used Muxing with condition to drive.

3. Combinational Loop:

A signal feeds itself without sequential elements → unstable hardware. The output of a combinational circuit will be feedback as an input to that combinational circuit without going through any sequential element.

Example :  assign a = b;

assign b = a;  // FATAL — loop
Risk: Oscillation or undefined convergence.
Fix: Need to Insert flop or break dependency. Best practice is see the schematic and follow the loop path to know which signal making combinational loop and from where you can break the path or make it delay.
NOTE: When you insert the Flop you will introduce a delay of one clock and that may cause issues.

4. Undeclared or Implicit Net (if default_nettype not none):
If you reference something that doesn't exist or maybe you forgot to declare it. Typo creates implicit wire.
Risk: Silent functional bug.

Fix: Search for that signal in the file using (gvim => ‘/name_signal ‘ and multiple search ‘/n’ ) , make sure it is declared properly.

5. Array / Index Out-of-Bounds

Illegal or negative indexing. Index exceeds declared range.

Ex: logic [7:0] data;
                     assign x = data[9]; // Out of range

assign x = data[-1];   //No hardware equivalent.

Risk:   Non-synthesizable, Tool-dependent simulation behaviour and can collapse to constant or X.

Fix: Fix the width of index.

6. Clock / Reset Structural Integrity Violations

Using non-static expressions for clocks, Gating clocks in unintended ways or Driving clock or reset with combinational logic.

Ex: always @(posedge (clk & en))  // Clock edge is generated using combinational logic.

Risk:  Logic-derived clock introduces Spurious edges, Pulse shrinking, Hold violations and Double-triggering of flops.

Fix:  always @(posedge clk)
           if(en)
           q <= d;

7.Unconnected Critical Ports

Required ports left floating like Unconnected clocks and Unconnected resets.

Risk: Unconnected clocks cause Flops never triggered, Clock tree synthesis fails and  Entire sequential logic becomes inactive.

Escalated to Fatal when reset drives FSM, Control logic not self-initializing and Safety-critical block.
Fix: Intentional Tie-Off and Explicitly Mark Unused Ports.

8. Unsupported or Dangerous System Tasks in Synthesizable RTL

force / release , $random inside synthesizable logic or Lint flags these as FATAL if found in synthesizable code.

Risk: No hardware equivalent.

Synthesis ignores it.

RTL sim behavior ≠ gate-level netlist behavior.

Can mask real structural bugs.

Fix: Avoid this all in any RTL Design.

9. Write-Write Conflict in Memory

Example:

always @(posedge clk)
                        if(we1)
                  mem[addr] <= d1;

                  always @(posedge clk)
                          if(we2)
                  mem[addr] <= d2;

Risk: Undefined memory content or Arbitration not defined in RTL.

Fix: Merge Writes Into Single Procedural Block (Priority-Based) or Arbitration Logic Before Memory

10. Mixed Async Reset Polarity on Same Register Bank

Ex:  always @(posedge clk or negedge rst_n)

Somewhere else in same register bank

always @(posedge clk or posedge rst_n)

Fix: Use same clock edge in all places don’t mix the edges.

ERROR VIOLATIONS: Synthesizable but unsafe / ambiguous

These are violations that do not break elaboration i.e synthesizable , but cause functional bugs, synthesis mismatches, or unpredictable behaviour. This is kind a rule breaking.

- Inferred latch:
Incomplete assignment in combinational block, as no else condition on if-else or no default case in case statements.
Risk: Unintended level-sensitive latch, Timing unpredictability or STA complexity

Fix: Be sure to have else condition in if-else and default in case

Ex: Problem: always @(*) begin
                                  if (en)
                                  y = a;
                            end

- Flop Without Reset in Control Path:
Sequential logic missing reset.

Risk: Random power-up state, FSM illegal encoding or Non-deterministic startup
Fix: Never miss  Reset in sequential block also it should have the highest priority.
Imp: Datapath flops may not require reset — control flops must.

- Blocking Assignment in Sequential Logic:
Sequential block is using blocking assignment instead of non-blocking assignment.
Risk: Simulation executes sequentially, Hardware updates in parallel or Sim–Synth mismatch.

Fix: Always use the non- blocking assignment in sequential block and blocking in combinational block.

Ex: Problem: always @(posedge clk) begin
                                           a = b;
                                            b = a;
                              end

- Undriven Net / Floating Signal :

A declared signal that is never assigned.

Risk: X-propagation, Synthesis may tie it to 0/1 silently and RTL vs gate-level mismatch

Fix: Assign in all code paths, Tie-off intentionally or Remove unused signals

- Unconnected Output Ports:

Outputs that no logic uses or no module connects.

Risk: Missing integration-level connectivity, Functional behaviors silently dropped or Synthesis removes logic → mismatch

Fix: Connect the output or If truly unused → comment or remove

- Incomplete Sensitivity List :
the sensitivity list the always block doesn’t include all the input in which output is depended on

Risk: Simulation mismatch vs synthesis or Glitches or Missed updates

Fix: Use always_comb (system Verilog ) , use (*) in sensitivity list or ensure all inputs are included

- Unconnected Output Ports

Outputs that no logic uses or no module connects.

Risk: Missing integration-level connectivity

Functional behaviors silently dropped

Synthesis removes logic → mismatch

Fix: Connect the output

If truly unused → comment or remove

Solution:    always @(*) begin
                               y = 0;
                                 if (en)          y = a;
                        end

Solution: always @(posedge clk) begin
                                           a <= b;
                                            b <= a;
                              end

8. Unreachable State or Unreachable Code

Typically found in FSM or branching logic.

Risk: FSM incomplete, Logic never activated or Testbench coverage holes

Fix: Remove or correct state transitions

9. Assignment to Output Port in Multiple Blocks

Not multiple drivers (FATAL), but incorrect drive methodology.

Ex:  output logic y;

always_ff @(posedge clk) y <= a;

always_comb y = b; // ERROR (not FATAL)
    Risk: Non-deterministic or Synthesis collapse

Fix: Decide single driver or use internal signals

10. Width Mismatch (Non-Catastrophic)

If the mismatch is not catastrophic enough to be FATAL.

Ex: logic [7:0] a;

logic [3:0] b;

assign b = a; // upper bits truncated → ERROR

Risk: Truncation, Zero-extension or Synthesis mismatch

Fix: Resize signals, Mask explicitly or Use $bits()

WARNINGS:  Warnings indicate potential issues, not immediate failures.
These do not cause simulation crashes or synthesis errors, but must be reviewed because they lead to:

- suboptimal quality

- verification difficulties

- future functional bugs

- hard-to-maintain code

1. Unused Signal / Unused Variable.
        Risk: Dead code, Confusion for new developers or Possible integration oversight

Fix: Remove the signal or Or prefix it with _unused_ if intentionally left

2.  Unreachable Code / Unreachable State

Tool detects logic or states that cannot be activated.
        Risk: Poor code hygiene, Unrealistic testbench coverage or Potential missing conditions

Fix: Remove unreachable blocks or Fix logic conditions

3. Truncated or Extended Assignment (Soft Width Warning)

Non-critical width mismatch. In which left hand width is much more than right assigning signal width.

Risk: Misleading logic or Poor clarity

Fix: Resize explicitly

4. Constant Conditional or Constant Loop Warning
      Ex: if (0) begin

...

end
     Risk: Dead logic or Redundant area if synthesis doesn't prune

Fix: Refactor conditions

5. Naming Conventions / Style Guide Violations

Tools check for:

- ALL CAPS for parameters

- lower_case for signals

- Appropriate name for module names

- snake_case for ports

- prefixing resets with rst_

Risk: Reduces readability or Non-standard coding style

Fix: Follow team’s style guide

Several lint issues have severity that depends on the data type or operand interpretation.

1. Signed / Unsigned Arithmetic Mismatch: Dependency on operand sign
     Ex: logic signed [7:0] a;
            logic [7:0] b;
            logic [7:0] y;
            assign y = a + b; // Result casting rules may produce unexpected arithmetic.

| Condition | Severity |
| --- | --- |
| mixed signed/unsigned but safe width | WARNING |
| mixed signed/unsigned causing sign-extension risk | ERROR |

2. Width Mismatch (Truncation / Extension): Dependency on operand bit-width
     Ex: logic [15:0] a;
            logic [7:0] b;
            assign b = a;   // Upper 8 bits truncated

| Condition | Severity |
| --- | --- |
| intentional truncation (constant) | WARNING |
| runtime variable truncation | ERROR |

3. 2-State vs 4-State Type Mixing :Dependency on data type (bit vs logic)
     Ex: bit a;
            logic b;
            assign a = b;           // logic can have X/Z but bit cannot represent X/Z

| Condition | Severity |
| --- | --- |
| no X/Z propagation possible | WARNING |
| possible X/Z loss | ERROR |

4. Packed vs Unpacked Assignment: Dependency or array type
     Ex: logic [7:0] a;

logic b[7:0];

assign a = b;              // Structural mismatch or Synthesis ambiguity.

| Condition | Severity |
| --- | --- |
| legal cast exists | WARNING |
| implicit illegal conversion | ERROR |

5. Comparison Between Different Types: Dependency on signed and width

Ex: logic signed [7:0] a;
            logic [7:0] b;

if (a < b)             // Comparison uses unsigned conversion.

| Condition | Severity |
| --- | --- |
| explicit cast present | WARNING |
| implicit conversion | ERROR |

6. Case Expression Width Mismatch: Dependency on case expression width
      Ex: logic [3:0] sel;

case(sel)

2'b00: ...

2'b01: ...

endcase

| Condition | Severity |
| --- | --- |
| value still reachable | WARNING |
| case item unreachable | ERROR |

Waiving Lint Error
A lint waiver suppresses a rule violation when the RTL intentionally deviates from a lint guideline, and the behavior is verified to be safe.

Purpose:

- Avoid false positives

- Maintain clean lint reports

- Document design intent

- Prevent unnecessary fixes that may impact functionality

Errors should only be waived when functional correctness is guaranteed.
Common valid cases:
1) Naming convention broke for clock and reset:
     Lint will issue Error when any signals which is clock but the name is not ended  with “*_clk” (similarly for rst )

2) Intentional Width Truncation:
     Lint may flag width mismatch, but the truncation is intentional.
justification: Lower byte extraction from 32-bit bus.
                           Upper bits intentionally ignored.

3) Constant Tie-Off Signals
                Lint rule may flag unused output or constant driver.

Example: assign debug_port = 1'b0;

Justification: Debug interface reserved for future feature.

How Waivers Are Implemented
Example waiver entry: rule WIDTH_MISMATCH

file alu.v

line 124

justification "Intentional truncation of lower byte"
Syntax: waive -rule WIDTH_MISMATCH -file alu.v -line 124     (SpyGlass)
Best practice:
1) Never waive blindly: Each waiver must include technical justification.
2) Do not waive FATAL errors.
3) Use centralized waiver files: Avoid inline suppression when possible.
4) Review waivers periodically: Some waivers become unnecessary after design updates.

Recommended Waiver Documentation Format

For project documentation:

| Field | Description |
| --- | --- |
| Rule ID | Lint rule name |
| File | RTL file |
| Line | Line number |
| Reason | Technical justification |
| Owner | Responsible engineer |

Follow the path:   RTL → Lint → Fix Errors → Review Warnings → Apply Waivers → Clean Report

Linting Limitations :
1) Lint tools analyse code structure, not functional behavior.

2) Cannot Detect Protocol Violations, ex:  SDA changing while SCL is HIGH (I2C violation) or SPI sampling on wrong edge

3) Lint does not perform real timing analysis. Ex: setup, hold violation or clock skew problem

4) Lint cannot predict runtime conditions like: Rare state transitions, Corner-case sequences or Data-dependent bugs

5) Cannot Replace Formal Verification, Ex: FIFO never overflows, FSM never enters illegal state

Or Deadlock-free arbitration