Reset Domain Crossing (RDC)
Reset Domain Crossing is the phenomenon that occurs when a signal passes from logic controlled by one reset into logic controlled by another reset, and these resets are asynchronous to each other or to the destination clock. Can happen in both synchronous and asynchronous clock domains.

This is not about clock mismatch (CDC problem). Instead, it’s about reset release timing mismatch.

Reset recovery time (trec):  It refer to time between when reset is de-asserted and the time the clock signal goes active again (like setup time). Minimum amount of time before the rising edge of clock for which reset should be stable, so that due to reset data doesn’t change at the active clock edge and flip flop goes to metastability .

Reset removal time (trem): It refer to time between when the time the clock signal goes active, and reset is de-asserted  (like hold time). Minimum amount of time after the rising edge of clock for which reset should be stable.

| Aspect | CDC (Clock Domain Crossing) | RDC (Reset Domain Crossing) |
| --- | --- | --- |
| Source of issue | Asynchronous clocks | Asynchronous reset de-assertion |
| When it occurs | During normal signal transfer | During reset release (initialization) |
| Risk | Wrong data sampled / metastability | Metastability / illegal state / chip may not boot |
| Visibility in sim | Sometimes seen (X-propagation) | Often hidden (X resolved) |
| MTBF | Low mean time between failure | High mean time between failure |
| Severity | Functional bugs (localized) | Chip-killer bug (global, harder to debug) |

CDC bugs corrupt data paths &  RDC bugs corrupt reset behaviour → may stop chip from coming out of reset at all.

That’s why RDC is considered more dangerous than CDC.

rstn_2

rstn_1

Din

clk

D2               Q2

D1               Q1

clk

Din

rstn_1

Q1

rstn_2

Q2_old	Q2_new

trec   trem

www.linkedin.com/in/harshita-harshi-9377891bb

Solution:

- Reset synchronizer circuit: Use a two-flop synchronizer between logic controlled by different resets to safely transfer signals.

clk

Din

rstn_1

Q1

rstn_2

Q’

Q’’

Q2

clk

Din

D``          Q``

D`              Q`

D2                    Q2

rstn_1

rstn_2

D1               Q1

sync_ff2

sync_ff1

module rdc_synchronizer (

input  clk,    // source clock

input   rst1_n,     // source reset (active-low)

input  Din,       // input data (from source domain)

input rst2_n,     // destination reset (active-low)

output reg  Q2       // synchronized output

);

reg Q1;

always @(posedge clk or negedge rst1_n) begin

if (!rst1_n)

Q1 <= 1'b0;

else

Q1 <= Din;

end

// Destination domain 2-flop synchronizer

reg sync_ff1 ,sync_ff2;

always @(posedge clk or negedge rst2_n)

begin

if (!rst2_n) begin

sync_ff1 <= 1'b0;

sync_ff2<=1'b0;

Q2   <= 1'b0;

end else begin

sync_ff1 <= Q1;

sync_ff2<= sync_ff1;

Q2    <= sync_ff2;

end

end

endmodule

The source flop Q1 is controlled by rst1_n, while the destination flops (sync_ff1, sync_ff2, and Q2) are controlled by rst2_n. If rst1_n deasserts earlier, Q1 starts capturing input data, but the destination logic remains held in reset by rst2_n, preventing unsafe propagation. When rst2_n finally deasserts, the destination flops begin operation and capture the Q1 output through the 2-flop synchronizer, which provides one extra cycle for metastability to settle before reaching Q2. In this way, reset release in one domain does not corrupt the other, and any reset recovery/removal timing violations are absorbed safely inside the synchronizer, ensuring a clean and metastability-free output.

www.linkedin.com/in/harshita-harshi-9377891bb

Even though the destination may not capture the source value immediately, the two-flip-flop synchronizer ensures metastability is confined inside the synchronizer chain and does not propagate into the rest of the design.

- Reset Ordering:
Reset ordering means defining and enforcing the sequence in which different resets in a system are asserted and deasserted.
In a multi-clock or multi-power-domain SoC, you may have:

- Global reset (Power-on reset / watchdog reset).

- Subsystem resets (CPU, bus, peripherals, memories).

- Functional resets (peripheral soft reset via software).

These must not all come out of reset simultaneously, because some blocks depend on others. So, with the counter we set when should and which reset should be on.

module reset_ordering #(parameter DELAY1 = 4,parameter DELAY2 = 6)

(

input  clk,

input  async_rst_n,          // global async reset

output logic rst1_n,        // reset 1

output logic rst2_n         // reset 2

);

typedef enum logic [1:0] {RST_ALL, RST1, RST2, DONE} state_t;

state_t state, next_state;

int cnt;

// Sequential state

always_ff @(posedge clk or negedge async_rst_n) begin

if (!async_rst_n) begin

state <= RST_ALL;

cnt   <= 0;

end else begin

state <= next_state;

cnt   <= (state != next_state) ? 0 : cnt + 1;

end

end

always_comb begin

next_state = state;

case (state)

RST_ALL: if (async_rst_n) next_state = RST1;

RST1:    if (cnt >= DELAY1) next_state = RST2;

RST2:    if (cnt >= DELAY2) next_state = DONE;

DONE:    next_state = DONE;

endcase

end

// Reset outputs (active low)

assign rst1_n = (state >= RST1);

assign rst2_n = (state >= RST2);

endmodule

!async_rst_n
!

RST_ALL

async_rst_n

!async_rst_n

async_rst_n

DONE

RST1

if (cnt >= DELAY2)

if (cnt >=DELAY1)

RST2

www.linkedin.com/in/harshita-harshi-9377891bb

- Handshake / toggle:

When you need a deterministic, acknowledged event across domains (e.g., domain A must finish init, then domain B may start), use a toggle/request-ack handshake. This guarantees detection and avoids races.

Source toggles a single-bit req_toggle each time it wants to signal an event. Target synchronizes the toggle and detects an edge to create a one-cycle event_pulse. Target then toggles ack_toggle back; source synchronizes ack and knows the event was received.

module reset_handshake (

input  logic clk,

input  logic async_rst_n,   // Global async reset

// Acknowledgment from block1

input  logic ack1,

output logic rst1_n,        // Reset for block1

output logic rst2_n         // Reset for block2

);

typedef enum logic [1:0]

{RST_ALL, RST1, RST2, DONE} state_t;

state_t state, next_state;

always_ff @(posedge clk or negedge async_rst_n)

begin

if (!async_rst_n)

state <= RST_ALL;

else

state <= next_state;

end

always_comb begin

next_state = state;

case (state)

RST_ALL: next_state = RST1;

RST1:    if (ack1) next_state = RST2;

RST2:    next_state = DONE;

DONE:    next_state = DONE;

endcase

end

assign rst1_n = (state != RST_ALL);  // rst1 released first

assign rst2_n = (state == RST2 || state == DONE);  // rst2 released after ack1

endmodule

www.linkedin.com/in/harshita-harshi-9377891bb

!async_rst_n

ack1

async_rst_n

RST1

RST2

DONE

RST_ALL