AMBA 2

AMBA High-performance Bus (AHB): A high-bandwidth, pipelined bus protocol. It supports single-cycle bus master handover, burst transfers, and split transactions, providing higher efficiency. Unlike APB, it is suitable for performance-critical system paths. AHB enabled better scalability in SoCs, but as multi-core systems and advanced IPs demanded even greater throughput and flexibility, AHB was eventually surpassed by AXI.

AMBA 1

Advanced Peripheral Bus (APB): A simple, low-power bus optimized for connecting peripherals. It uses a non-pipelined protocol with minimal control signals, making it easy to implement and energy-efficient. it offers easy integration but was replaced in high-performance paths due to limited speed and bandwidth.

Advanced System Bus (ASB): A high-performance, pipelined bus that improved speed and efficiency over simpler buses, but it was eventually replaced as systems demanded greater bandwidth, scalability, and advanced transaction handling.

AMBA 5

Adaptive Traffic Profiles (ATP): A standard for modeling and generating realistic traffic patterns to analyze and verify interconnect performance (like AXI/CHI). It helps designers test bandwidth, latency, and QoS requirements early in SoC design.

Coherent Hub Interface (CHI): A packet-based, cache-coherent interconnect protocol for many-core SoCs, offering high scalability, efficiency, and replacing AXI+ACE in modern high-performance designs.
AHB-Lite: A simplified version of AHB that supports only a single master, while retaining burst transfers, pipelining, and high bandwidth. It reduces complexity and cost, making it ideal for microcontrollers and embedded systems.

AMBA 4

AXI Coherency Extensions (ACE): ACE adds cache coherency support across multiple processors and accelerators in a system. It enables CPUs and other coherent masters to share memory consistently without software-driven cache maintenance. ACE introduces new signals and transactions (such as snoop requests and responses) to manage cache-to-cache communication, invalidations, and updates, ensuring all agents see the latest data.

AXI4: It supports high-performance, burst-based transactions, separate read/write channels, multiple outstanding transactions, and out-of-order completion, making it ideal for CPU–memory and high-speed interconnects

AMBA 3

Advanced eXtensible Interface (AXI): AXI is a high-performance, high-bandwidth interconnect protocol, introduces separate read and write channels, supports out-of-order transactions, burst transfers, and multiple outstanding addresses, which allow parallelism and reduce latency. AXI provides scalability and Quality of Service (QoS) features for prioritization.

Advanced Trace Bus (ATB): It is a dedicated trace bus used to transport debug and trace data from multiple trace sources to trace sinks. It enables real-time monitoring and debugging without disturbing the normal system operation. ATB supports high-speed, low-latency data transfer.

AHB-Lite: A simplified version of the AHB that supports only a single bus master. It retains the key features of AHB like burst transfers, pipelined operation, and high bandwidth while reducing complexity and hardware cost by removing multi-master arbitration logic.

2014-
2016-

2019

2011-

2010

2003

1999

1990

### AMBA (Advanced Microcontroller Bus Architecture)

### Arm Advanced Microcontroller Bus Architecture, or AMBA, is an open-standard, on-chip interconnect specification for the connection and management of functional blocks in system-on-a-chip (SoC) designs

- Efficient IP reuse - IP reuse is an essential component in reducing SoC development costs and timescales.

- Flexibility - flexibility to work with a range of SoCs with different power, performance, and area requirements.

- Compatibility - allows compatibility between IP components from different design teams or vendors.

- Support - widely implemented and supported throughout the semiconductor industry.

| Protocol | Purpose / Use-case | Key Features | Limitations | Current Status |
| --- | --- | --- | --- | --- |
| APB (Advanced Peripheral Bus) | Low-speed peripherals (GPIO, UART, Timers) | Simple, low-power, easy integration, non-pipelined | Very low bandwidth, no burst | Still widely used for peripheral access |
| ASB (Advanced System Bus) | Early system bus for CPUs ↔ memory/peripherals | Pipelining, multiple masters, burst transfers | Limited scalability, replaced by AHB/AXI | Obsolete |
| AHB (AMBA High-performance Bus) | High-speed system bus (CPU ↔ memory, DMA) | Burst transfers, pipelining, split transactions, single-cycle master handover | Less scalable for multi-core | Replaced by AXI in high-performance SoCs |
| AHB-Lite | Embedded/microcontrollers with one master | Same as AHB but single master only, simpler and cheaper | No multi-master support | Still used in MCUs and small SoCs |
| AXI (Advanced eXtensible Interface) | High-performance interconnect (CPU, GPU, memory, DMA) | Separate read/write channels, multiple outstanding transactions, out-of-order, QoS | Complex implementation | Industry standard in modern SoCs |
| AXI4 | AMBA 4 generation, memory-mapped, streaming, and lite variants | Burst-based, scalable, supports AXI4-Lite (control), AXI4-Stream (streaming) | Complexity overhead for small designs | Widely adopted |
| ACE (AXI Coherency Extensions) | Multi-core cache-coherent systems | Adds cache snoop transactions, coherency between masters | Higher complexity and wiring overhead | Used but being replaced by CHI |
| ATB (Advanced Trace Bus) | Debug & trace data transport | High-speed, non-intrusive trace support (CoreSight) | Not for normal data transfers | Still used in ARM debug architecture |
| ATP (Adaptive Traffic Profiles) | Traffic modeling & verification | Realistic, parameterized traffic patterns to test performance | Not a data bus (only for simulation/verification) | Used in verification flows |
| CHI (Coherent Hub Interface) | Many-core, high-performance SoCs | Packet-based, scalable, full cache coherency, virtualization, flow control | Complex protocol, requires advanced fabric | Latest standard replacing ACE/AXI in high-end SoCs |