## QUESTA CDC TOOL

Clock Domain Crossing (CDC) Verification:
Clock Domain Crossing (CDC) verification identifies unsafe signal transfers between asynchronous or unrelated clock domains.
CDC tools analyze the RTL structure to detect violations that may cause:

- metastability

- data incoherency

- reconvergence hazards

- synchronization failures

- reset crossing issues

Possible consequences:

- incorrect logic evaluation

- intermittent silicon failures

- difficult-to-reproduce bugs

- field failures

CDC issues are rare in simulation but catastrophic in silicon, which is why CDC verification is mandatory before RTL signoff.

Input Files Required for Questa CDC: CDC tools require both design RTL and intent information.

1)  RTL Design Files: RTL files describe the design logic and module hierarchy with module definitions, signal declarations, clocked logic and  combinational logic.

2) Constraint File: “constraints.do”

- Clock Definition: CDC tools must know which signals are clocks.

“ set clock_name     TIME_PERIOD”
 “ netlist clock path_of_clk   -period TIME_PERIOD   -waveform {low_time     high_time}  -group NAME_CLK”

- Constant signal Constraints: Some signals which need constant value to execute, like atpg_mode select, testmodes.
“netlist constant path_with.signal_name 1'b0”

- I/O Constraints: Define the input/output signals belong to which clock domain.
“netlist port domain i_o_NAME   -input -clock CLK_NAME”
“netlist port domain I_o_NAME   -input   -async”

3)Exceptions File : “exceptions.do”
    Define the stable signals when its sure to be stable.
         “cdc signal -stable path.sig_name”
  - registers input signals are stable
  - some signals which you already assign the two sync

4) Clock Relationship Constraints: Tools must know if clocks are related or asynchronous.

“set_clock_group -asynchronous {clk_a clk_b}”
       means: clk_a and clk_b have no timing relationship
 All circuit will have basis clock i.e internal clock and external clock (protocol clock)and the signal directly coming to circuit as input are async.

Din                               comb

clk1                                          clk2

CDC Severity condition:

- Violation: A violation is a must-fix problem. The crossing is not an instance of a valid CDC scheme.

Violation indicates the CDC path does not match any recognized safe CDC structure and must be fixed or waived.

- All the signals coming directly as an input to digtop module (Top module), if not sync properly surely a violation.

- All analog signals coming to digital are async, need to sync with internal clock.

- Combinational logic between domains without sync 
example :
 outp = a +b;  a in clk1 and b in clk2 
 so, outp will have CDC issues.
solution: Make both the inputs in same clock by converting any of the variable using synchronizer to the clock decided.

- Reconvergence without proper synchronization

- Async reset crossing incorrectly handled

Examples that cause Violation

- Single flop synchronizer

- Direct combinational CDC

- Multi-bit bus without handshake/FIFO

- Async reset deassertion unsynchronized

- Clock gating logic crossing domains

- Reconvergent CDC path without sync

- Caution (Warning Type): Structure correct but protocol must be verified

The tool reports Caution when Crossing matches a known CDC scheme, but protocol correctness is not        guaranteed, requires protocol checking (simulation/formal)

Examples : These are valid CDC structures, but protocol errors like:

- Handshake CDC (req/ack) : req without ack

- FIFO CDC : FIFO overflow/underflow

- Pulse synchronizer : Pulse width too small, can still cause issues → so tool gives Caution, not Violation.

- Toggle synchronizer

- Bus with enable control

- Gray code pointer crossing

- Evaluation: Valid CDC structure detected

- Crossing matches a valid CDC scheme

- Tool recognizes it as safe structurally

- Protocol checker not fully proven yet

- Proven (Best Case): Fully verified CDC crossing.

- CDC structure valid

- Protocol formally proven safe

- No protocol violation possible

Questa CDC assigns severity based on whether a crossing matches a recognized CDC synchronization scheme and whether the transfer protocol is formally proven. Crossings that do not match valid CDC structures are reported as Violations, while crossings that match valid CDC schemes but require protocol verification are reported as Caution.

Safe Waivers conditions:
1) Mutually exclusive clock domain crossing :
       Example : outp = a & b;  a  in Clk1 and b in Clk2 ;
       Then outp will may show CDC issues, but if when this equation is going to execute  only one clock is enable and other clk is stable or gated off then this will be safe condition.

2) Intentional CDC crossing

Example:   Test mode signals

Scan enable

Debug signals

Configuration registers written once

Interrupt signals

Status signals

These may cross clock domains without synchronizer but are functionally safe.

2. Reset crossings: Async reset assertion is OK, but deassertion may be flagged.
          If reset tree is verified separately → waiver.

3. Static signals:
     Signals that: Change very rarely

Remain constant after configuration

Sampled only when stable

Tool flags them, but they are safe → Waiver.

Example: mode_select
                          config_bus
                          test_enable

4. Synchronizer recognized but tool cannot detect properly

Sometimes tool reports violation even though:

Double flop synchronizer exists

But due to hierarchy/logic, tool cannot infer

5. Reconvergence but functionally safe

Tool may flag reconvergence violation, but if designer proves:

Signals synchronized properly

No functional hazard

Example Waiver Justification

Signal          : cfg_mode[2:0]
         Crossing    : clk_cfg → clk_core
         Type             : Static configuration bus
         Reason      : Written once after reset, remains constant during operation
         Safety         : Sampled only when cfg_done asserted
       Verification: Verified in simulation and design review

Sceniros

1)You have a 4-bit control bus crossing from a 200MHz domain to a 50MHz domain. A colleague suggests using a 4-bit 2FF synchronizer.

What is wrong with this approach, and what is the correct solution? Explain the failure mode precisely.

My Answer: 2 flip flop synchronizers are used for single bit transfer not for bus, for bus transfer we should use either handshaking mechanics or FIFO

Follow up questions: What is the actual failure mechanism? What physically happens when you put a 4-bit bus through 4 independent 2FF synchronizers?

Think about this scenario:

- Bit 0 of your bus transitions at time T

- Bit 3 of your bus transitions at time T+200ps (same clock edge, but routed differently — different wire length, different cell delay)

Both bits go through their own independent 2FF synchronizers

What does the receiving domain see? What is the exact name of this problem?

My Answer: for the scenario you given, you propose each bit with 2ff but in this way the bit transfer are independent of each other and because of delays there will be chance of Data mismatch, questions on Data integrity and data loose.
Correct response: Data mismatch due to independent sampling is the core problem. But I want you to use the precise industry term for this.

That failure mode has a specific name. It is called metastability propagation leading to bus incoherency — but more specifically the problem you described is called bit skew or multi-bit CDC violation.

Each bit samples at a different moment in the destination domain. So the receiver captures a torn value — some bits from the old value, some bits from the new value. Neither the old state nor the new state. A completely invalid intermediate state that corrupts your system.

2)When do you choose handshaking over FIFO for multi-bit CDC? What is the deciding factor?

I will give you a concrete scenario to anchor your thinking:

- Scenario A: A 32-bit configuration register written occasionally by software, read by hardware in another clock domain

- Scenario B: A continuous stream of 16-bit ADC samples flowing from a 100MHz capture domain into a 250MHz processing domain

Which mechanism do you use for each, and why? What is the fundamental property of each scenario that drives your decision?

Answer:

The real deciding factor is not speed. It is data production rate vs consumption rate and whether data can be lost or must be buffered.

In Scenario A — a configuration register — data changes rarely and sporadically. There is no stream. The sender produces one value, the receiver must capture exactly that value, and then nothing happens for milliseconds. Handshaking works here because you need guaranteed delivery of a single coherent value, not throughput. The latency of req/ack is acceptable because writes are infrequent.

In Scenario B — ADC samples — data is produced every single clock cycle at 100MHz. The sender cannot stop. It does not wait for acknowledgment. If you use handshaking, you will drop samples because the req/ack round trip across clock domains takes multiple cycles. A FIFO absorbs the rate difference and the timing uncertainty between domains. It decouples producer from consumer permanently.

3)   Why Gray code specifically? What property of Gray code makes it safe for CDC?

  What happens if you used binary encoded pointers instead across the clock domain boundary?

  Your FIFO has depth 8. Write pointer is at binary 011 moving to 100. Show me exactly why this is catastrophic with binary encoding across C
Answer:Your answer on Gray code — what you got right and what is missing:

You said "only one bit flips" — correct. That is the key property.

But you missed why that matters specifically in a CDC context. Let me complete your reasoning:

In a CDC crossing, the receiving flip-flop samples the incoming signal at an arbitrary moment. If only one bit is transitioning, there are only two possible outcomes when the receiver samples — it captures the old value or the new value. Both are valid pointer values. The FIFO may appear one position ahead or behind momentarily, but it never generates a phantom address that does not exist.

With binary encoding, multiple bits transition simultaneously. The receiver can sample during the transition of different bits at different moments. It does not capture old or new — it captures a combination that was never a valid pointer value. That phantom address points to a location that was never written. You read garbage data or falsely declare full/empty.

DC, and why Gray code solves it.
Binary 011 → 100 — all three bits flip simultaneously.

The receiver could sample:

- Bit 2 = 1 (new), Bit 1 = 0 (old), Bit 0 = 0 (old) → reads 100 = 4 ✓ valid but wrong timing

- Bit 2 = 0 (old), Bit 1 = 1 (new), Bit 0 = 1 (new) → reads 011 = 3 ✓ also valid

- Bit 2 = 1 (new), Bit 1 = 1 (new), Bit 0 = 0 (old) → reads 110 = 6 — this address never existed in this transition

- Bit 2 = 0 (old), Bit 1 = 0 (old), Bit 0 = 1 (new) → reads 001 = 1 — also never existed

That 110 or 001 phantom pointer is catastrophic. Your empty/full logic makes a decision based on an address that was never valid. Data corruption or false full/empty — silent bug in silicon.

Now Gray code for the same transition:

Binary 3 (011) in Gray = 010 Binary 4 (100) in Gray = 110

Only bit 2 flips — from 0 to 1. The receiver samples either 010 or 110. Both are valid Gray code pointer values. No phantom state is possible.